ACOUSTIC WEIGHTING NETWORK MODULE AWN-3

FEATURES

- Simultaneous A, B and C outputs

- ANSI S1, 42-1986 (R1998) Type 0

- Miniature plug-in module

- ± 3.5V to ± 15V supply operation

- Milled aluminium construction

- One year warranty

- Development board available

GENERAL DESCRIPTION

The AWN-3 is an encapsulated filter module offering A, B and C weighting networks to Type 0 precision. The AWN-3 is designed for use in OEM applications where a plug-in acoustic weighting network is required.

The device offers simultaneous A, B and C weighted outputs with a ± 2.5V signal range that is suitable for most modern instrumentation. Power requirement is a dual, low current        ± 3.5V to ± 15V supply. On board regulation and reverse voltage protection are provided.

For stand-alone and experimental applications, a development board DEV-1 is available for the AWN-3 module providing convenient power supply and signal connections.

OUTLINE DIMENSIONS

a

e

e

f

b

d

d

c

a = 46 mm

b = 20 mm

c = 5.5 mm

d = 19 mm

e = 9.5 mm

f  = 14 mm

Pins 0.64 mm square

PIN CONNECTIONS

Pin 1 = V+ (Positive supply)

Pin 2 = 0V (Supply reference)

Pin 3 = V-  (Negative supply)

Pin 4 = Output 1 (A-weighting)

Pin 5 = Output 2 (B-weighting)

Pin 6 = Output 3 (C-weighting)

Pin 7 = 0V (Supply Reference)

Pin 8 = Input (Maximum + 2.5V)